PARPORT TURBO PASCAL PARALLEL PORT I/O SUPPORT

Terry King   terry@terryking.us

152 Colby Road

West Topsham,  Vermont 05086

V1.20  02/07/05

These routines have been developed over several years of use with

school kids, primarily in grades 5 to 8.  The program that I put

together is referred to as "Bit & Bytes".  Sometimes kids get into

a longer-term project called "KidMachines" where they design and

build some funny computer-controlled machine.

PARPORT utilizes an IBM-compatible machines PRINTER PORT, often called

the PARALLEL PORT. PARPORT 'fixes' the funny way the bits and ports were

designed, specifically to control a printer. All you need to do is think

of your machine as having (1) An 8-bit OUTPUT port, with the 8 bits connected

to pins 2 thru 9 on the connector, and (2) An 8-bit INPUT port, with the 8

bits connected to pins 10 thru 17 on the connector (Pins 18 thru 25 are GROUND).

You've probably heard of how weird the parallel port is! It's true, but this

software fixes it by moving the funny input bits around and inverting those

that need it.

This level of software is for those who write their own programs in

Turbo Pascal (From Borland) and want to control the parallel port. If you're

just starting out and don't want to write any software YET, use SEEBITS.EXE

which is usually part of the ZIP file you got this software in. It is a

DOS program that shows you the state of the INPUT bits and allows you to

turn OUTPUT bits on and off by pushing keys on your keyboard.

NOTE: Which PORT is used by PARPORT may be determined in two ways:

1. Command Line: The parameter LPT1 or LPT2 or LPTM may be used on the

command line of your program. (Default is LPT2).

2. Environment Variable: you may set the PARPORT environment variable

either from the DOS prompt or in AUTOEXEC.BAT like this:

SET PARPORT=LPT1   (or LPT2 or LPTM) (See PARPORT.DOC for port addresses)

DOCUMENTATION IS IN 2 PARTS BELOW:

(1) A written description of each of the Procedures and Functions

(2) The formal Turbo Pascal Unit INTERFACE part. I supply this and the

compiled turbo PARPORT.TPU files so you can link this with your

own programs. I am not supplying the source code for the implementation

of PARPORT.

PART 1:     PARPORT  PROCEDURES AND FUNCTIONS:

---------------------------------

{-------------( Low-Level I/O )----------------------------}

PROCEDURE WriteDO(WriteData: byte);

WriteDO simply takes a byte in which YOU have decided what bits should

be on and off, and sends it out to the OUTPUT PORT.

If you created a data variable called "LEDS", you would write a

program line like this:

WriteDo(LEDS);

FUNCTION  ReadDI: byte;

ReadDI reads in a byte from the INPUT PORT. It assigns the data it

got back to a variable you have defined. (It fixes up the 'funny'

parallel port bits, as mentioned above).

If you created a data variable called "SWITCHES", you would write a

program line like this:

SWITCHES := ReadDI;

PROCEDURE DioReset;

DioReset simply writes ZEROS to the OUTPUT port: often a good thing

to do first in your program.

{-------------( Input and Bit Sensing )--------------------}

PROCEDURE CheckSensors;

CheckSensors is usually used in a loop in your program. It reads in

the current state of the input bits and saves that value in a global

variable called SenseBitsNow.  This enables you to use some easy

Functions like ButtonPushed without worrying about WHICH bit it is.

(See the PROGRAM EXAMPLE in the file TRYPPORT.PAS)

FUNCTION  ButtonPushed(ButtonId: INBITPOS): Boolean;

(You have to have called CheckSensors for ButtonPushed to work.)

ButtonPushed checks to see if a specific Bit is ON in the INPUT PORT.

It returns a Boolean (TRUE or FALSE) result.

You use it in a program like this:

IF ButtonPushed(SusysButton) THEN Say('Hi Susy.');

PROCEDURE WaitButtonPushed(ButtonId: INBITPOS);

WaitButtonPushed checks and KEEPS CHECKING to see if a specific

Bit is ON in the INPUT PORT. WHEN the bit comes on THEN it continues

and your program continues. CAUTION: WaitButtonPushed will wait FOREVER

if the bit never comes on!

You use it in a program like this:

WaitButtonPushed(SusysButton);

...Then your program continues...

{-------------( Individual Bit Output )--------------------}

These each turn a single bit ON or OFF on the OUTPUT PORT.  You SHOULD set

up a CONSTANT in your program for each bit you want to use, so you can just

use the NAME of the bit, not its Bit Number in the port. These were just made

up for it to be easy for kids to write software.

Example:

CONST

RedLight   = 0;     {Red LED connected to Bit 0 of OUTPUT PORT. (Pin 2) }

GreenLED   = 1;     {Green LED connected to Bit 1 of OUTPUT PORT. (Pin 3) }

Beeper     = 6;     {Beeper device connected to Bit 6 of OUTPUT PORT. (Pin 8) }

AirBalloon = 7;     {Compressed air valve controlling blowing up balloon(Pin 9)}

(Now, you can use the procedures below, like this):

BitOn(RedLight);       {Turn on the Red Light Emitting Diode}

TurnOn(RedLight);      {Does exactly the same. Just an 'alias'}

LightOn(RedLight);     {Does exactly the same. Just an 'alias'}

LightOn(GreenLED);     {etc etc}

LightOff(RedLight);    {Turn OFF the Red Light Emitting Diode}

TurnOff(Beeper);       {etc etc}

Blink(RedLight, 4, 1000); {Turn Red LED on & off 4 times, 1 second each}

Pouf(AirBalloon, 500); {Turn on Air Valve for 500/1000 of a second}

PART 2:     PARPORT  INTERFACE DEFINITION:

-----------------------------

UNIT PARPORT;

{---------------------------------------------------------------------------}

{ TURBO PASCAL SUPPORT FOR PARALLEL PRINTER PORT FOR GENERAL PURPOSE I/O    }

{                                                                           }

{01/17/94 Original creation                                                 }

{01/19/94 Updates to handle funny input register bits, create 8 bit Input   }

{09/03/96 Update: Get Base Addr from command line: LPT1, LPT2, LPTM         }

{01/27/97 Update: 1 second delay to see port address                        }

{02/01/97 Clean up vestigal stuff...                                        }

{03/02/97 Change DIInvert to $92 (Had a mistake I guess!)                   }

{                                                                           }

{ NOTE: This software is copyright 1995, 1996, 1997 by Terry King           }

{                                                                           }

{       Terry King    terry@terryking.us                                     }

{                                                                           }

{---------------------------------------------------------------------------}

INTERFACE

(*�������������������������������������������������������������������������Ŀ

� INTERFACE PART OF UNIT: PUBLIC OBJECTS ARE HERE:                        �

� -- USES (External Public Objects)                                       �

� -- CONST, TYPE, VAR                                                     �

� -- PROCEDURE, FUNCTION                                                  �

���������������������������������������������������������������������������*)

USES HexUtil,  {Conversion utilities}

Dos,Crt;  {Regular Borland UNITS }

{------( PUBLIC CONSTANTS )------------------------}

CONST

CopyRight   = 'PARPORT Copyright (c) 1996, 1997 By Terry King. tking@together.net ' +

'Box 633 Shelburne, Vermont 05482';

AUTHOR        =  'Terry King Box 633 Shelburne, VT 05482 tking@together.net';

MAXBITPOS     =  7;              {Max Bit position in a 8-bit byte }

{------( PUBLIC TYPES     )------------------------}

TYPE

OUTBITPOS     =  0..7;              {Type for a Bit position in a 8-bit byte }

INBITPOS      =  0..7;              {Type for a Bit position in PAR PORT input}

VAR   {------( PUBLIC VARIABLES )------------------------}

BitsNowOn    : byte;{Global holder of current state of Digital Output Reg.}

SenseBitsNow : byte;{Global holder of last read of Sensors/Buttons    }

{NOTE: Set by any call to CheckSensors }

PortAddrString : String;   {String showing port address being used}

(*�������������������������������������������������������������������������Ŀ

� PROCEDURES AND FUNCTIONS TO BE CALLED EXTERNALLY:                       �

�                                                                         �

���������������������������������������������������������������������������*)

{-------------( Low-Level I/O )----------------------------}

PROCEDURE WriteDO(WriteData: byte);

FUNCTION  ReadDI: byte;

PROCEDURE DioReset;

{-------------( Input and Bit Sensing )--------------------}

PROCEDURE CheckSensors;

FUNCTION  ButtonPushed(ButtonId: INBITPOS): Boolean;

PROCEDURE WaitButtonPushed(ButtonId: INBITPOS);

{-------------( Individual Bit Output )--------------------}

PROCEDURE BitOn(BitToSet: OUTBITPOS);       { These are all aliases }

PROCEDURE BitOff(BitToReset: OUTBITPOS);    {}

PROCEDURE TurnOn(BitToSet: OUTBITPOS);      {}

PROCEDURE TurnOff(BitToSet: OUTBITPOS);     {}

PROCEDURE LightOn(Light: OUTBITPOS);        {}

PROCEDURE LightOff(Light: OUTBITPOS);       {}

PROCEDURE Blink(Light: OUTBITPOS; NumTimes: Integer; TimeOn: Integer );

PROCEDURE Pouf(Pouffer: OUTBITPOS; HowLong: word);

{--( END OF INTERFACE PART )-------------------------------------------------}

#